TITLE 36

LEGISLATIVE RULE

BOARD OF COAL MINE HEALTH AND SAFETY

SERIES 26

RULES AND REGULATIONS GOVERNING CONSTRUCTION AND

REHABILITATION OPERATIONS IN UNDERGROUND MINES

'36-26-1.  General.

1.1.  Scope.  --  Rules and Regulations Governing Construction and Rehabilitation Operations in Underground Mines.

1.2.  Authority.  --  W. Va.  Code '22-6-4.

1.3.  Filing Date.  --  May 25, 1984.

1.4.  Effective Date.  --  July 1, 1984.

'36-26-2.  Effect of Regulations.

2.1.  These rules and regulations shall have the effect of law, and violations shall be deemed a violation of law and so cited with the same effect as law.  All provisions of Chapter 22A, Article 1A of the West Virginia Code relative to enforcement of these rules and regulations.

'36-26-3.  Definitions.

3.1.  All terms used in these rules and regulations, not defined herein, shall have the meanings set forth in Chapter 22 A, Article 1A, Section 1 of the West Virginia Code.

'36-26-4.  On-shift Examination of Construction and Rehabilitation Work Areas.

4.1.  In addition to the pre-shift examination, an on-shift examination shall be made of all construction and rehabilitation work areas between the third and fifth hours while persons are working, and such examination shall be recorded in a book prescribed for such purposes, and located on the surface.

'36-26-5.  Automated Temporary Roof Support (ATRS) Systems on Roof Bolting Machines used for Construction and Rehabilitation.

5.1.  Where required by the Director of the Department of Energy, roof bolting machines used for construction or for rehabilitation work shall be equipped with an approved automated temporary roof support (ATRS) system.  However, when spot bolting is done on track haulage roads, machines shall not be required to have ATRS.

5.2.  In determining where such ATRS system is needed, the Director shall consider the following: (a) whether the use of ATRS would pose a greater hazard than not using it; (b) whether the technology of ATRS exists to allow compliance with these requirements; and (c) the conditions at the construction or rehabilitation site.

5.3.  Minimum Requirements for Machines Using, or Used as, ATRS.  All machines using, or used as, an automated temporary roof support system in accordance with this section shall comply with the minimum requirements for ATRS as specified in West Virginia Board of Coal Mine Health and Safety, Series 10, Section 5.3.