TITLE 36

LEGISLATIVE RULE

BOARD OF COAL MINE HEALTH AND SAFETY

SERIES 42

SEALING PERMANENTLY CLOSED OR ABANDONED MINES

'36-42-1.  General.

1.1.  Scope.  --  Rules and regulations governing sealing permanently closed or abandoned mines.

1.2.  Authority.  --  W. Va. Code '22-6-4.

1.3.  Filing Date.  --  January 9, 1995.

1.4.  Effective Date.  --  July 1, 1995.

'36-42-2.  Effective Law and Regulation.

2.1.  These regulations shall have the effect of the law and violations shall be deemed a violation of law and so cited with the same effect as law.  All provisions of Chapter 22, Article 1 of the West Virginia Code relative to enforcement are applicable to the enforcement of these regulations.

'36-42-3.  Definitions.

All terms used in these rules and regulations, not defined herein, shall have the meaning set forth in Section one, Article one A, Chapter twenty-two A of the Code.

'36-42-4.  Sealing or Capping.

4.1.

(a)  Prior to the sealing or capping of any shaft, slope or drift opening, the operator must submit to the Director for approval its written plan for the safe sealing or capping.  No sealing or capping may commence without written approval from the Director.  Once submitted, the approval plan required by this section shall not be unreasonably withheld.  If the Director does not approve such plan, he shall provide the operator, in writing, reasons for rejecting the plan.  At a minimum, the plan must contain:

(1)  detailed map(s) of the mine area to be abandoned;

(2)  detailed drawings of the seal or cap design;

(3)  the projected time-table for commencement and completion of the sealing or capping;

(4)  the procedures to be used to adequately ventilate the areas to be sealed or capped while the work is conducted; and

(5)  Efforts should be taken to limit cutting and welding during sealing and capping operations however, precaution shall be taken if cutting and welding is used, including methane examinations underneath the cap.