Title 36

Legislative Exempt Rule

Board of Coal Mine Health & Safety

Series 60

Rules and Regulations Governing Equipment and Operations for Open-Pit Mines, the Surface of Underground Mines, Preparation Plants and Loadouts

§36-60-1.  General.

1.1. Scope. -- Rule governing equipment requirements and operations for open-pit mines, the surface of underground mines, preparation plants and loadouts.

1.2. Authority. -- W. Va. Code §§ 22A-6-4, 22A-6-5, 22A-4-2.

1.3. Filing Date. --

1.4. Effective Date. -- To be determined after notice and comment period.

§36-60-2.  Effect of Rule.

2.1.  This rule shall have the effect of law and violations shall be deemed a violation of law and so cited with the same effect as law. All provisions of W. Va. Code § 22A-1-1 et seq. relative to enforcement are applicable to the enforcement of this rule.

§36-60-3.  Definitions.

3.1.  All terms used in this rule, not defined herein, shall have the meanings set forth in W. Va. Code §§ 22A-1-2, 22A-4-1.

§36-60-4.  General Requirements.

4.1.  The following requirements apply to equipment and operations in open-pit mines, surface of underground mines, preparation plants and loadouts:

4.1.1.  When maintenance is being performed in an area other than a designated maintenance area, the operator is required to:

4.1.1.a.  Develop a safety zone around the equipment 10 ft. from the furthest point the equipment could reach.  This safety zone may implement the use of physical barriers such as road cones, signage, flashing strobes or any combination of those or other devices that would clearly identify the safety zone.

4.1.1.b.  Limit the personnel in that area except those needed to perform the task.

4.1.1.c.  Require that all people working in that area have a two-way radio.

4.1.1.d.  Require that between the time 30 minutes before sunset to 30 minutes after sunrise, all personnel working in the area must have a cap light or strobe light on.

4.1.2.  All personnel while out of equipment must wear reflective material that can be seen from all directions on the outermost garment from the waist up.