TITLE 60

INTERPRETIVE RULE

DEPARTMENT OF ENVIRONMENTAL PROTECTION, SECRETARY’S OFFICE

SERIES 9

VOLUNTARY REMEDIATION AND REDEVELOPMENT DE MINIMIS STANDARDS

§60-9-1.  General.

1.1.  Scope.  --  This Interpretive Rule establishes the De Minimis Standards applicable for voluntary remediation activities and brownfield revitalization performed in accordance with the Voluntary Remediation and Redevelopment Act, W. Va. Code § 22-22-1, et seq.

1.2.  Authority.  --  W. Va. Code § 22-22-3.

1.3.  Filing Date.  --  July 30, 2021.

1.4.  Effective Date.  --  August 29, 2021.

§60-9-2.  Definitions.

The definitions contained in W. Va. Code § 22-22-2 and the Voluntary Remediation and Redevelopment Rule, 60CSR3, apply to this rule, in addition to those definitions set forth below:

2.1.  “De Minimis Standard” means a contaminant level that poses no substantial risk to human health based on the current or reasonably anticipated future land and groundwater use for either residential or industrial purposes.

§60-9-3.  De Minimis Standards.

3.1.  The Secretary shall review the De Minimis Standards annually and, if necessary, update the De Minimis Standards to reflect current toxicity information, chemical-specific data, and exposure parameters.  Additionally, updates to the De Minimis Standards shall occur following significant regulatory changes such as a revised groundwater quality standard or listing of a new hazardous substance in § 104(14) of CERCLA.

3.2.  The Voluntary Remediation and Redevelopment Rule, 60CSR3 (§ 9.2.d), outlines the method for calculating De Minimis Standards.

3.3.  De Minimis Standards to be used in all voluntary remediation activities and brownfield revitalization are presented in Table 60-9.

Table 60-9:  De Minimis Standards Table

| CONTAMINANT | CAS No. | Residential Soil1,3 (mg/kg) | Value Basis4 | Industrial Soil1,3 (mg/kg) | Value Basis4 | Ground Water2,3 (ug/L) | Value Basis4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acetaldehyde | 75-07-0 | 1.2E+01 1.0E+01 | c | 3.7E+02 | nc | 2.6E+00 2.2E+00 | c |
| Acetochlor | 34256-82-1 | 1.3E+03 | nc | 1.6E+04 | nc | 3.5E+02 | nc |
| Acetone | 67-64-1 | 6.1E+04 7.0E+04 | nc | 1.1E+05 | Csat | 1.4E+04 1.8E+04 | nc |
| Acetonitrile | 75-05-8 | 8.7E+02 | nc | 3.7E+03 | nc | 1.3E+02 | nc |
| Acetophenone | 98-86-2 | 2.5E+03 | Csat | 2.5E+03 | Csat | 1.9E+03 | nc |
| Acrolein | 107-02-8 | 1.5E-01 | nc | 6.5E-01 | nc | 4.2E-02 | nc |
| Acrylamide | 79-06-1 | 2.4E-01 | c | 4.6E+01 | c | 5.0E-02 4.8E-02 | c |
| Acrylonitrile | 107-13-1 | 2.7E-01 2.4E-01 | c | 1.2E+01 | c | 5.2E-02 4.6E-02 | c |
| Alachlor | 15972-60-8 | 9.7E+00 9.2E+00 | c | 4.1E+02 | c | 2.0E+00 | gws |
| Alar | 1596-84-5 | 3.0E+01 2.9E+01 | c | 1.3E+03 | c | 4.3E+00 3.8E+00 | c |
| Aldicarb | 116-06-3 | 6.3E+01 | nc | 8.2E+02 | nc | 3.0E+00 | gws |
| Aldicarb sulfone | 1646-88-4 | 6.3E+01 | nc | 8.2E+02 | nc | 2.0E+00 | gws |
| Aldrin | 309-00-2 | 3.9E-02 3.7E-02 | c | 1.8E+00 | c | 9.2E-04 8.0E-04 | c |
| Aluminum | 7429-90-5 | 7.7E+04 | nc | 1.0E+06 | max | 2.0E+04 | nc |
| Aniline | 62-53-3 | 9.5E+01 9.0E+01 | c | 4.0E+03 | c | 1.3E+01 1.2E+01 | c |
| Antimony and compounds | 7440-36-0 | 3.1E+01 | nc | 4.7E+02 | nc | 6.0E+00 | gws |
| Arsenic | 7440-38-2 | 6.8E-01 3.0E-02 | c | 3.0E+01 1.4E+00 | c | 1.0E+01 | gws |
| Assure | 76578-14-8 | 5.7E+02 | nc | 7.4E+03 | nc | 1.2E+02 | nc |
| Atrazine | 1912-24-9 | 2.4E+00 2.2E+00 | c | 1.0E+02 | c | 3.0E+00 | gws |
| Azobenzene | 103-33-3 | 5.6E+00 5.3E+00 | c | 2.6E+02 | c | 1.2E-01 1.0E-01 | c |
| Barium and compounds | 7440-39-3 | 1.5E+04 | nc | 2.2E+05 | nc | 2.0E+03 | gws |
| Baygon | 114-26-1 | 2.5E+02 | nc | 3.3E+03 | nc | 7.8E+01 | nc |
| Baythroid | 68359-37-5 | 1.6E+03 | nc | 2.1E+04 | nc | 1.2E+02 | nc |
| Bentazon | 25057-89-0 | 1.9E+03 | nc | 2.5E+04 | nc | 5.7E+02 | nc |
| Benzaldehyde | 100-52-7 | 1.7E+02 | c | 1.2E+03 | Csat | 1.9E+01 1.6E+01 | c |
| Benzene | 71-43-2 | 1.2E+00 1.1E+00 | c | 5.4E+01 | c | 5.0E+00 | gws |
| Benzidine | 92-87-5 | 5.3E-04 5.2E-04 | c | 1.0E-01 | c | 1.0E-04 | c |
| Benzoic acid | 65-85-0 | 2.5E+05 | nc | 1.0E+06 | max | 7.5E+04 | nc |
| Benzyl alcohol | 100-51-6 | 6.3E+03 | nc | 8.2E+04 | nc | 2.0E+03 | nc |
| Benzyl chloride | 100-44-7 | 1.1E+00 1.0E+00 | c | 5.1E+01 | c | 8.9E-02 7.8E-02 | c |
| Beryllium and compounds | 7440-41-7 | 1.6E+02 | nc | 2.3E+03 | nc | 4.0E+00 | gws |
| 1,1'-Biphenyl | 92-52-4 | 5.1E+01 | nc | 2.1E+02 | nc | 8.3E-01 | nc |
| Bis(2-chloroethyl)ether | 111-44-4 | 2.4E-01 2.2E-01 | c | 1.1E+01 | c | 1.4E-02 1.2E-02 | c |
| Bis(2-chloroisopropyl)ether | 108-60-1 | 5.1E+00 4.7E+00 | c | 2.3E+02 | c | 3.6E-01 3.1E-01 | c |
| Bis(chloromethyl)ether | 542-88-1 | 8.9E-05 7.7E-05 | c | 3.9E-03 | c | 7.2E-05 6.3E-05 | c |
| Bis(2-ethylhexyl)phthalate (DEHP) | 117-81-7 | 3.9E+01 3.7E+01 | c | 1.6E+03 | c | 6.0E+00 | gws |
| Bromodichloromethane | 75-27-4 | 3.1E-01 2.7E-01 | c | 1.4E+01 | c | 1.3E-01 8.0E+01 | gws |
| Bromoform (tribromomethane) | 75-25-2 | 2.0E+01 1.8E+01 | c | 9.1E+02 | c | 3.3E+00 8.0E+01 | gws |
| Bromomethane | 74-83-9 | 7.3E+00 | nc | 3.2E+01 | nc | 7.5E+00 | nc |
| Bromophos | 2104-96-3 | 3.4E+02 | nc | 3.8E+03 3.9E+03 | nc | 1.8E+01 1.9E+01 | nc |
| 1,3-Butadiene | 106-99-0 | 8.1E-02 7.1E-02 | c | 3.6E+00 | c | 7.1E-02 6.2E-02 | c |
| 1-Butanol | 71-36-3 | 4.7E+03 4.9E+03 | nc | 7.6E+03 | Csat | 5.3E+02 5.9E+02 | nc |
| Butylate | 2008-41-5 | 3.2E+03 3.3E+03 | nc | 3.3E+04 3.4E+04 | nc | 2.1E+02 2.2E+02 | nc |
| n-Butylbenzene | 104-51-8 | 1.1E+02 | Csat | 1.1E+02 | Csat | 1.0E+03 | nc |
| Butyl benzyl phthalate | 85-68-7 | 2.9E+02 2.7E+02 | c | 1.2E+04 | c | 1.6E+01 1.4E+01 | c |
| Cadmium and compounds | 7440-43-9 | 3.7E+01 7.5E+00 | nc | 5.3E+02 1.1E+02 | nc | 5.0E+00 | gws |
| Caprolactam | 105-60-2 | 3.1E+04 | nc | 4.0E+05 | nc | 9.9E+03 | nc |
| Carbaryl | 63-25-2 | 6.3E+03 | nc | 8.2E+04 | nc | 1.8E+03 | nc |
| Carbon disulfide | 75-15-0 | 7.4E+02 | Csat | 7.4E+02 | Csat | 8.1E+02 | nc |
| Carbon tetrachloride | 56-23-5 | 7.0E-01 6.1E-01 | c | 3.1E+01 | c | 5.0E+00 | gws |
| Carbosulfan | 55285-14-8 | 6.3E+02 | nc | 8.2E+03 | nc | 5.1E+01 | nc |
| Chloranil | 118-75-2 | 1.4E+00 1.3E+00 | c | 5.7E+01 | c | 1.8E-01 1.6E-01 | c |
| Chlordane (Technical Mixture) | 12789-03-6 | 1.9E+00 1.8E+00 | c | 8.9E+01 | c | 2.0E+00 | gws |
| Chloroacetic acid | 79-11-8 | 1.3E+02 2.2E+02 | nc | 1.6E+03 2.9E+03 | nc | 4.0E+01 6.0E+01 | ncgws |
| 4-Chloroaniline | 106-47-8 | 2.7E+00 2.6E+00 | c | 1.1E+02 | c | 3.7E-01 3.2E-01 | c |
| Chlorobenzene | 108-90-7 | 2.9E+02 | nc | 7.6E+02 | Csat | 1.0E+02 | gws |
| Chlorobenzilate | 510-15-6 | 4.9E+00 4.7E+00 | c | 2.1E+02 | c | 3.1E-01 2.8E-01 | c |
| p-Chlorobenzoic acid | 74-11-3 | 1.9E+03 | nc | 2.5E+04 | nc | 5.1E+02 | nc |
| 2-Chloro-1,3-butadiene (chloroprene) | 126-99-8 | 1.1E-02 3.7E-03 | c | 4.7E-01 | c | 1.9E-02 6.4E-03 | c |
| 1-Chlorobutane | 109-69-3 | 2.5E+02 | nc | 7.3E+02 | Csat | 2.0E+02 | nc |
| Chloroethane | 75-00-3 | 2.1E+03 | Csat | 2.1E+03 | Csat | 2.1E+04 8.3E+03 | nc |
| Chloroform | 67-66-3 | 3.4E-01 3.0E-01 | c | 1.5E+01 | c | 8.0E+01 | cgws |
| Chloromethane | 74-87-3 | 1.2E+02 | nc | 5.0E+02 | nc | 1.9E+02 | nc |
| 4-Chloro-2-methylaniline | 95-69-2 | 5.4E+00 5.2E+00 | c | 2.3E+02 | c | 7.0E-01 6.2E-01 | c |
| beta-Chloronaphthalene | 91-58-7 | 5.0E+03 5.1E+03 | nc | 5.0E+04 5.3E+04 | nc | 3.3E+02 3.5E+02 | nc |
| o-Chloronitrobenzene | 88-73-3 | 1.8E+00 1.7E+00 | c | 7.7E+01 | c | 2.4E-01 2.1E-01 | c |
| p-Chloronitrobenzene | 100-00-5 | 9.0E+00 8.6E+00 | c | 3.8E+02 | c | 1.2E+00 1.0E+00 | c |
| 2-Chlorophenol | 95-57-8 | 3.4E+02 3.5E+02 | nc | 3.9E+03 4.0E+03 | nc | 2.7E+01 2.9E+01 | nc |
| o-Chlorotoluene | 95-49-8 | 4.5E+02 5.0E+02 | nc | 9.1E+02 | Csat | 9.0E+01 9.8E+01 | nc |
| Chlorpyrifos-methyl | 5598-13-0 | 6.3E+02 | nc | 8.2E+03 | nc | 1.2E+02 | nc |
| Chromium III | 16065-83-1 | 1.2E+05 4.9E+04 | nc | 1.0E+06 3.0E+05 | maxnc | 2.2E+04 | nc |
| Chromium VI | 18540-29-9 | 3.1E-01 9.4E-01 | c | 6.3E+01 2.0E+02 | c | 3.5E-02 1.0E-01 | c |
| Cobalt | 7440-48-4 | 2.3E+01 | nc | 3.5E+02 | nc | 6.0E+00 | nc |
| Copper and compounds | 7440-50-8 | 3.1E+03 | nc | 4.7E+04 | nc | 8.0E+02 1.3E+03 | ncgws |
| Crotonaldehyde | 123-73-9 | 8.2E-02 8.0E-02 | c | 3.6E+00 4.0E+00 | c | 8.2E-03 7.9E-03 | c |
| Cyanazine | 21725-46-2 | 6.5E-01 6.1E-01 | c | 2.7E+01 | c | 8.8E-02 7.7E-02 | c |
| Cyanide and compounds | 74-90-8 | 2.3E+01 | nc | 1.5E+02 | nc | 2.0E+02 | gws |
| Cyanogen | 460-19-5 | 7.8E+01 | nc | 1.2E+03 | nc | 2.0E+01 | nc |
| Cyanogen bromide | 506-68-3 | 7.0E+03 | nc | 1.1E+05 | nc | 1.8E+03 | nc |
| Cyclohexane | 110-82-7 | 1.2E+02 | Csat | 1.2E+02 | Csat | 1.3E+04 | nc |
| Cyclohexanone | 108-94-1 | 5.1E+03 | Csat | 5.1E+03 | Csat | 1.4E+03 | nc |
| Cyhalothrin/Karate | 68085-85-8 | 6.3E+01 | nc | 8.2E+02 | nc | 2.0E+01 | nc |
| Cypermethrin | 52315-07-8 | 1.3E+03 4.6E+03 | nc | 1.6E+04 5.9E+04 | nc | 4.0E+02 1.4E+03 | nc |
| Dacthal | 1861-32-1 | 6.3E+02 | nc | 8.3E+03 8.2E+03 | nc | 1.2E+02 | nc |
| Dalapon | 75-99-0 | 1.9E+03 | nc | 2.5E+04 | nc | 2.0E+02 | gws |
| DDD | 72-54-8 | 1.9E+00 2.1E+00 | ncc | 2.5E+01 9.6E+01 | ncc | 3.2E-02 2.8E-02 | c |
| DDE | 72-55-9 | 2.0E+00 1.9E+00 | c | 9.3E+01 | c | 4.6E-02 4.0E-02 | c |
| DDT | 50-29-3 | 1.9E+00 1.8E+00 | c | 8.5E+01 | c | 2.3E-01 2.0E-01 | c |
| Diazinon | 333-41-5 | 4.4E+01 | nc | 5.7E+02 | nc | 1.0E+01 | nc |
| Dibenzofuran | 132-64-9 | 7.8E+01 | nc | 1.2E+03 | nc | 7.9E+00 | nc |
| 1,4-Dibromobenzene | 106-37-6 | 4.1E+02 4.4E+02 | nc | 2.8E+03 3.1E+03 | nc | 4.7E+01 5.1E+01 | nc |
| Dibromochloromethane | 124-48-1 | 8.3E+00 7.9E+00 | c | 3.9E+02 | c | 8.7E-01 8.0E+01 | cgws |
| 1,2-Dibromo-3-chloropropane | 96-12-8 | 5.7E-03 5.4E-03 | c | 6.9E-01 | c | 2.0E-01 | gws |
| 1,2-Dibromoethane | 106-93-4 | 3.9E-02 3.4E-02 | c | 1.7E+00 | c | 5.0E-02 | gws |
| Dibutyl phthalate | 84-74-2 | 6.3E+03 | nc | 8.2E+04 | nc | 9.0E+02 | nc |
| Dicamba | 1918-00-9 | 1.9E+03 | nc | 2.5E+04 | nc | 5.7E+02 | nc |
| 1,2-Dichlorobenzene | 95-50-1 | 3.8E+02 | Csat | 3.8E+02 | Csat | 6.0E+02 | gws |
| 1,4-Dichlorobenzene | 106-46-7 | 2.8E+00 2.4E+00 | c | 1.2E+02 | c | 7.5E+01 | gws |
| 3,3'-Dichlorobenzidine | 91-94-1 | 1.2E+00 1.1E+00 | c | 5.1E+01 | c | 1.3E-01 1.1E-01 | c |
| 1,4-Dichloro-2-butene | 764-41-0 | 2.3E-03 2.0E-03 | c | 1.0E-01 | c | 1.3E-03 1.2E-03 | c |
| Dichlorodifluoromethane | 75-71-8 | 9.4E+01 | nc | 4.0E+02 | nc | 2.0E+02 | nc |
| 1,1-Dichloroethane | 75-34-3 | 3.8E+00 3.3E+00 | c | 1.7E+02 | c | 2.8E+00 2.4E+00 | c |
| 1,2-Dichloroethane | 107-06-2 | 5.0E-01 4.3E-01 | c | 2.2E+01 | c | 5.0E+00 | gws |
| 1,1-Dichloroethylene | 75-35-4 | 2.4E+02 5.1E+00 | nc | 1.1E+03 2.2E+01 | nc | 7.0E+00 | gws |
| 1,2-Dichloroethylene (cis) | 156-59-2 | 1.7E+01 6.5E+01 | nc | 8.0E+01 3.9E+02 | nc | 7.0E+01 | gws |
| 1,2-Dichloroethylene (trans) | 156-60-5 | 7.5E+01 | nc | 3.2E+02 | nc | 1.0E+02 | gws |
| 2,4-Dichlorophenol | 120-83-2 | 1.9E+02 | nc | 2.5E+03 | nc | 4.6E+01 | nc |
| 4-(2,4-Dichlorophenoxy)butyric Acid (2,4-DB) | 94-82-6 | 1.9E+03 | nc | 2.5E+04 | nc | 4.5E+02 | nc |
| 2,4-Dichlorophenoxyacetic Acid (2,4-D) | 94-75-7 | 7.0E+02 | nc | 9.6E+03 | nc | 7.0E+01 | gws |
| 1,2-Dichloropropane | 78-87-5 | 2.7E+00 2.3E+00 | c | 7.1E+01 | nc | 5.0E+00 | gws |
| 1,3-Dichloropropene | 542-75-6 | 1.9E+00 1.7E+00 | c | 8.6E+01 | c | 4.7E-01 4.1E-01 | c |
| 2,3-Dichloropropanol | 616-23-9 | 1.9E+02 | nc | 2.5E+03 | nc | 5.9E+01 | nc |
| Dichlorvos | 62-73-7 | 1.9E+00 1.8E+00 | c | 7.9E+01 | c | 2.6E-01 2.3E-01 | c |
| Dicyclopentadiene | 77-73-6 | 1.4E+00 | nc | 5.8E+00 | nc | 6.3E-01 | nc |
| Dieldrin | 60-57-1 | 4.2E-02 4.0E-02 | c | 2.0E+00 | c | 7.2E-04 6.3E-04 | c |
| Diethylene glycol, monobutyl ether | 112-34-5 | 1.9E+03 | nc | 2.4E+04 | nc | 6.0E+02 | nc |
| Diethylene glycol, monoethyl ether | 111-90-0 | 3.8E+03 | nc | 4.8E+04 | nc | 1.2E+03 | nc |
| Di(2-ethylhexyl)adipate | 103-23-1 | 4.5E+02 4.3E+02 | c | 1.9E+04 | c | 4.0E+02 | gws |
| Diethyl phthalate | 84-66-2 | 5.1E+04 | nc | 6.6E+05 | nc | 1.5E+04 | nc |
| Diethylstilbestrol | 56-53-1 | 1.6E-03 1.5E-03 | c | 6.6E-02 | c | 5.1E-05 4.5E-05 | c |
| Difenzoquat (Avenge) | 43222-48-6 | 5.2E+03 | nc | 6.8E+04 | nc | 1.7E+03 | nc |
| 1,1-Difluoroethane | 75-37-6 | 1.4E+03 | Csat | 1.4E+03 | Csat | 8.3E+04 | nc |
| Diisopropyl methylphosphonate | 1445-75-6 | 5.3E+02 | Csat | 5.3E+02 | Csat | 1.6E+03 | nc |
| 3,3'-Dimethoxybenzidine | 119-90-4 | 3.4E-01 7.5E-02 | c | 1.4E+01 | c | 4.7E-02 1.5E-02 | c |
| N-N-Dimethylaniline | 121-69-7 | 2.6E+01 2.5E+01 | c | 7.2E+02 7.8E+02 | nc | 2.5E+00 2.2E+00 | c |
| 2,4-Dimethylaniline | 95-68-1 | 2.7E+00 2.6E+00 | c | 1.1E+02 | c | 3.7E-01 3.3E-01 | c |
| 2,4-Dimethylaniline hydrochloride | 21436-96-4 | 9.4E-01 8.9E-01 | c | 4.0E+01 | c | 1.3E-01 1.2E-01 | c |
| 3,3'-Dimethylbenzidine | 119-93-7 | 4.9E-02 1.1E-02 | c | 2.1E+00 | c | 6.5E-03 2.0E-03 | c |
| 2,4-Dimethylphenol | 105-67-9 | 1.3E+03 | nc | 1.6E+04 | nc | 3.6E+02 | nc |
| 2,6-Dimethylphenol | 576-26-1 | 3.8E+01 | nc | 4.9E+02 | nc | 1.1E+01 | nc |
| 3,4-Dimethylphenol | 95-65-8 | 6.3E+01 | nc | 8.2E+02 | nc | 1.8E+01 | nc |
| 4,6-Dinitro-o-cyclohexyl phenol | 131-89-5 | 1.3E+02 | nc | 1.6E+03 | nc | 2.3E+01 | nc |
| 1,2-Dinitrobenzene | 528-29-0 | 6.3E+00 | nc | 8.2E+01 | nc | 1.9E+00 | nc |
| 1,3-Dinitrobenzene | 99-65-0 | 6.3E+00 | nc | 8.2E+01 | nc | 2.0E+00 | nc |
| 1,4-Dinitrobenzene | 100-25-4 | 6.3E+00 | nc | 8.2E+01 | nc | 2.0E+00 | nc |
| 2,4-Dinitrophenol | 51-28-5 | 1.3E+02 | nc | 1.6E+03 | nc | 3.9E+01 | nc |
| Dinitrotoluene (Technical Grade) | 25321-14-6 | 1.2E+00 1.1E+00 | c | 5.1E+01 | c | 1.0E-01 9.2E-02 | c |
| 2,4-Dinitrotoluene | 121-14-2 | 1.7E+00 | c | 7.4E+01 | c | 2.4E-01 2.1E-01 | c |
| 2,6-Dinitrotoluene | 606-20-2 | 3.6E-01 3.4E-01 | c | 1.5E+01 | c | 4.9E-02 4.3E-02 | c |
| Dinoseb | 88-85-7 | 6.3E+01 | nc | 8.2E+02 | nc | 7.0E+00 | gws |
| 1,4-Dioxane | 123-91-1 | 5.4E+00 5.0E+00 | c | 2.5E+02 | c | 4.6E-01 4.0E-01 | c |
| Diphenylamine | 122-39-4 | 6.3E+03 | nc | 8.2E+04 | nc | 1.3E+03 | nc |
| 1,2-Diphenylhydrazine | 122-66-7 | 6.8E-01 6.4E-01 | c | 2.9E+01 | c | 7.8E-02 6.9E-02 | c |
| Diquat | 85-00-7 | 1.4E+02 | nc | 1.8E+03 | nc | 2.0E+01 | gws |
| Disulfoton | 298-04-4 | 2.5E+00 | nc | 3.3E+01 | nc | 5.0E-01 | nc |
| 1,4-Dithiane | 505-29-3 | 5.4E+02 5.7E+02 | nc | 4.6E+03 4.9E+03 | nc | 5.3E+01 5.9E+01 | nc |
| Diuron | 330-54-1 | 1.3E+02 | nc | 1.6E+03 | nc | 3.6E+01 | nc |
| Endosulfan | 115-29-7 | 4.5E+02 | nc | 6.0E+03 6.1E+03 | nc | 3.1E+01 3.3E+01 | nc |
| Endothall | 145-73-3 | 1.3E+03 | nc | 1.6E+04 | nc | 1.0E+02 | gws |
| Endrin | 72-20-8 | 1.9E+01 | nc | 2.5E+02 | nc | 2.0E+00 | gws |
| Epichlorohydrin | 106-89-8 | 2.0E+01 | nc | 8.8E+01 | nc | 2.0E+00 | nc |
| Ethion | 563-12-2 | 3.2E+01 | nc | 4.1E+02 | nc | 4.3E+00 | nc |
| 2-Ethoxyethanol | 110-80-5 | 5.3E+03 2.7E+03 | nc | 4.9E+04 1.6E+04 | nc | 3.4E+02 8.0E+01 | nc |
| Ethyl acetate | 141-78-6 | 6.7E+02 | nc | 2.8E+03 | nc | 1.4E+02 | nc |
| Ethylbenzene | 100-41-4 | 6.2E+00 5.4E+00 | c | 2.7E+02 | c | 7.0E+02 | gws |
| Ethylene diamine | 107-15-3 | 6.3E+03 6.4E+03 | nc | 7.6E+04 7.8E+04 | nc | 4.9E+02 5.3E+02 | nc |
| Ethylene glycol | 107-21-1 | 1.3E+05 5.1E+04 | nc | 1.0E+06 6.6E+05 | maxnc | 4.0E+04 1.6E+04 | nc |
| Ethylene glycol, monobutyl ether | 111-76-2 | 6.3E+03 | nc | 8.2E+04 | nc | 2.0E+03 | nc |
| Ethylene thiourea (ETU) | 96-45-7 | 5.1E+00 | nc | 6.6E+01 | nc | 1.6E+00 1.5E+00 | nc |
| Ethyl ether | 60-29-7 | 2.1E+03 2.4E+03 | nc | 9.9E+03 1.0E+04 | nc | 1.1E+03 1.2E+03 | nc |
| Ethyl methacrylate | 97-63-2 | 1.1E+03 | Csat | 1.1E+03 | Csat | 6.3E+02 | nc |
| Fenamiphos | 22224-92-6 | 1.6E+01 | nc | 2.1E+02 | nc | 4.4E+00 | nc |
| Fluometuron | 2164-17-2 | 8.2E+02 | nc | 1.1E+04 | nc | 2.4E+02 | nc |
| Fluorine (Soluble Fluoride) | 7782-41-4 | 4.7E+03 | nc | 7.0E+04 | nc | 4.0E+03 | gws |
| Fomesafen | 72178-02-0 | 1.6E+02 6.3E+02 | nc | 2.1E+03 8.2E+03 | nc | 3.9E-01 1.9E+02 | nc |
| Fonofos | 944-22-9 | 1.3E+02 | nc | 1.6E+03 | nc | 1.9E+02 2.4E+01 | nc |
| Formaldehyde | 50-00-0 | 1.8E+01 3.6E+00 | c | 7.9E+02 5.8E+02 | c | 4.3E-01 1.5E-01 | c |
| Formic Acid | 64-18-6 | 3.1E+01 | nc | 1.3E+02 | nc | 6.3E-01 | nc |
| Furan | 110-00-9 | 9.1E+00 | nc | 4.2E+01 | nc | 5.3E+00 | nc |
| Furazolidone | 67-45-8 | 1.4E-01 | c | 6.0E+00 | c | 2.0E-02 1.8E-02 | c |
| Furfural | 98-01-1 | 2.2E+02 1.9E+01 | ncc | 2.7E+03 9.4E+02 | ncc | 3.8E+01 2.0E+00 | ncc |
| Glycidaldehyde | 765-34-4 | 2.4E+01 | nc | 2.1E+02 | nc | 1.7E+00 | nc |
| Glyphosate | 1071-83-6 | 6.3E+03 | nc | 8.2E+04 | nc | 7.0E+02 | gws |
| Heptachlor | 76-44-8 | 1.4E-01 1.3E-01 | c | 6.3+00 | c | 4.0E-01 | gws |
| Heptachlor epoxide | 1024-57-3 | 7.1E-02 6.7E-02 | c | 3.3E+00 | c | 2.0E-01 | gws |
| Hexabromobenzene | 87-82-1 | 1.5E+02 | nc | 2.0E+03 | nc | 1.1E+01 1.2E+01 | nc |
| Hexachlorobenzene | 118-74-1 | 2.2E-01 2.0E-01 | c | 1.0E+01 6.1E+00 | c | 1.0E+00 | gws |
| Hexachlorobutadiene | 87-68-3 | 1.3E+00 1.1E+00 | c | 1.7E+01 | Csat | 1.4E-01 1.2E-01 | c |
| HCH (alpha) | 319-84-6 | 8.6E-02 8.2E-02 | c | 3.6E+00 | c | 7.2E-03 6.4E-03 | c |
| HCH (beta) | 319-85-7 | 3.0E-01 2.9E-01 | c | 1.3E+01 | c | 2.5E-02 2.2E-02 | c |
| HCH (gamma) Lindane | 58-89-9 | 5.7E-01 5.7E-02 | c | 2.5E+01 8.0E-01 | c | 2.0E-01 | gws |
| HCH-technical | 608-73-1 | 3.0E-01 2.9E-01 | c | 1.3E+01 | c | 2.5E-02 2.2E-02 | c |
| Hexachlorocyclopentadiene | 77-47-4 | 1.9E+00 | nc | 8.0E+00 | nc | 5.0E+01 | gws |
| Hexachlorodibenzo-p-dioxin mixture (HxCDD) | Various | 1.0E-04 9.9E-05 | c | 4.7E-03 | c | 1.3E-05 1.1E-05 | c |
| Hexachloroethane | 67-72-1 | 2.0E+00 1.7E+00 | c | 8.6E+01 | c | 3.3E-01 2.9E-01 | c |
| Hexachlorophene | 70-30-4 | 1.9E+01 | nc | 2.5E+02 | nc | 6.0E+00 | nc |
| 1,6-Hexamethylene diisocyanate | 822-06-0 | 3.4E+00 | nc | 1.4E+01 | nc | 2.1E-02 | nc |
| n-Hexane | 110-54-3 | 1.4E+02 | Csat | 1.4E+02 | Csat | 1.5E+03 | nc |
| Hexazinone | 51235-04-2 | 2.1E+03 | nc | 2.7E+04 | nc | 6.4E+02 | nc |
| HMX | 2691-41-0 | 3.9E+03 | nc | 5.7E+04 | nc | 1.0E+03 | nc |
| Hydrazine | 302-01-2 | 3.4E-02 3.0E-02 | c | 1.5E+00 | c | 1.1E-03 9.5E-04 | c |
| Hydrogen sulfide | 7783-06-4 | 1.0E+06 | max | 1.0E+06 | max | 4.2E+00 | nc |
| p-Hydroquinone | 123-31-9 | 9.0E+00 8.6E+00 | c | 3.8E+02 | c | 1.3E+00 1.1E+00 | c |
| Iron | 7439-89-6 | 5.5E+04 | nc | 8.2E+05 | nc | 1.4E+04 | nc |
| Isobutanol | 78-83-1 | 1.0E+04 8.2E+03 | Csatnc | 1.0E+04 | Csat | 1.7E+03 7.3E+02 | nc |
| Isophorone | 78-59-1 | 5.7E+02 5.4E+02 | c | 2.4E+04 | c | 7.8E+01 6.9E+01 | c |
| Isopropalin | 33820-53-0 | 1.1E+03 | nc | 1.5E+04 | nc | 2.9E+01 3.0E+01 | nc |
| Isopropylbenzene (Cumene) | 98-82-8 | 2.7E+02 | Csat | 2.7E+02 | Csat | 4.5E+02 | nc |
| Isopropyl methyl phosphonic acid | 1832-54-8 | 6.3E+03 | nc | 8.2E+04 | nc | 2.0E+03 | nc |
| Lead | 7439-92-1 | 4.0E+02 2.0E+02 | nc | 8.0E+02 4.6E+02 | nc | 1.5E+01 1.0E+01 | gws |
| Lead (tetraethyl) | 78-00-2 | 7.8E-03 | nc | 1.2E-01 | nc | 1.3E-03 | nc |
| Lithium | 7439-93-2 | 1.6E+02 | nc | 2.3E+03 | nc | 4.0E+01 | nc |
| Malathion | 121-75-5 | 1.3E+03 | nc | 1.6E+04 | nc | 3.9E+02 | nc |
| Maleic anhydride | 108-31-6 | 6.3E+03 | nc | 8.0E+04 | nc | 1.9E+03 | nc |
| Manganese (non-food) | 7439-96-5 | 1.8E+03 | nc | 2.6E+04 | nc | 4.3E+02 | nc |
| Mephosfolan | 950-10-7 | 5.7E+00 | nc | 7.4E+01 | nc | 1.8E+00 | nc |
| Mepiquat | 24307-26-4 | 1.9E+03 | nc | 2.5E+04 | nc | 6.0E+02 | nc |
| Mercury (elemental and inorganic) | 7439-97-6 | 3.1E+00 | Csat | 3.1E+00 | Csat | 2.0E+00 | gws |
| Mercury (methyl) | 22967-92-6 | 7.8E+00 | nc | 1.2E+02 | nc | 2.0E+00 | nc |
| Methacrylonitrile | 126-98-7 | 7.6E+00 | nc | 1.0E+02 | nc | 1.9E+00 | nc |
| Methanol | 67-56-1 | 1.1E+05 | Csat | 1.1E+05 | Csat | 2.0E+04 | nc |
| Methidathion | 950-37-8 | 9.5E+01 | nc | 1.2E+03 | nc | 2.9E+01 | nc |
| Methoxychlor | 72-43-5 | 3.2E+02 | nc | 4.1E+03 | nc | 4.0E+01 | gws |
| Methyl acetate | 79-20-9 | 2.3E+04 2.5E+04 | nc | 2.9E+04 | Csat | 5.3E+03 5.9E+03 | nc |
| Methyl acrylate | 96-33-3 | 1.6E+02 | nc | 6.6E+02 | nc | 4.2E+01 | nc |
| Methyl Tertiary Butyl Ether (MTBE) | 1634-04-4 | 5.0E+01 4.4E+01 | c | 2.2E+03 | c | 1.4E+01 1.2E+01 | c |
| 2-Methylaniline (o-toluidine) | 95-53-4 | 3.4E+01 3.2E+01 | c | 1.4E+03 | c | 4.7E+00 4.2E+00 | c |
| 2-Methyl-4-chlorophenoxyacetic acid | 94-74-6 | 3.2E+01 | nc | 4.1E+02 | nc | 7.5E+00 | nc |
| 4-(2-Methyl-4-chlorophenoxy) butyric acid | 94-81-5 | 2.8E+02 2.8E+03 | nc | 3.6E+03 3.6E+04 | nc | 6.5E+01 6.5E+02 | nc |
| 2-(2-Methyl-4-chlorophenoxy) propionic acid | 93-65-2 | 6.3E+01 | nc | 8.2E+02 | nc | 1.6E+01 | nc |
| 4,4'-Methylenebisbenzeneamine | 101-77-9 | 3.4E-01 3.2E-01 | c | 1.4E+01 | c | 4.7E-02 4.2E-02 | c |
| 4,4'-Methylene bis(2-chloroaniline) | 101-14-4 | 1.2E+00 | c | 2.3E+02 | c | 1.6E-01 1.5E-01 | c |
| 4,4'-Methylene bis(N,N'-dimethyl)aniline | 101-61-1 | 1.2E+01 1.1E+01 | c | 5.0E+02 | c | 5.2E-01 6.6E-01 | c |
| Methylene bromide | 74-95-3 | 2.5E+01 | nc | 1.1E+02 | nc | 8.0E+00 | nc |
| Methylene chloride | 75-09-2 | 5.8E+01 5.7E+01 | c | 3.3E+03 | Csatnc | 5.0E+00 | gws |
| Methylenediphenyl diisocyanate | 101-68-8 | 8.5E+05 | nc | 1.0E+06 | max |  |  |
| Methyl ethyl ketone | 78-93-3 | 2.8E+04 | nc | 2.8E+04 | Csat | 5.6E+03 | nc |
| Methyl isobutyl ketone | 108-10-1 | 3.4E+03 | Csat | 3.4E+03 | Csat | 1.2E+03 | nc |
| Methyl methacrylate | 80-62-6 | 2.4E+03 | Csat | 2.4E+03 | Csat | 1.4E+03 | nc |
| 2-Methyl-5-nitroaniline | 99-55-8 | 6.0E+01 5.7E+01 | c | 2.6E+03 | c | 8.2E+00 7.2E+00 | c |
| Methyl parathion | 298-00-0 | 1.6E+01 | nc | 2.1E+02 | nc | 4.5E+00 | nc |
| 2-Methylphenol (o-Cresol) | 95-48-7 | 3.2E+03 | nc | 4.1E+04 | nc | 9.3E+02 | nc |
| 3-Methylphenol (m-Cresol) | 108-39-4 | 3.2E+03 | nc | 4.1E+04 | nc | 9.3E+02 | nc |
| 4-Methylphenol (p-Cresol) | 106-44-5 | 6.3E+03 1.3E+03 | nc | 8.2E+04 1.6E+04 | nc | 1.9E+03 3.7E+02 | nc |
| Methyl styrene (mixture) | 25013-15-4 | 3.3E+02 | nc | 3.9E+02 | Csat | 2.3E+01 | nc |
| Methyl styrene (alpha) | 98-83-9 | 5.0E+02 | Csat | 5.0E+02 | Csat | 7.8E+02 | nc |
| Metolaclor (Dual) | 51218-45-2 | 9.5E+03 | nc | 1.2E+05 | nc | 2.7E+03 | nc |
| Metribuzin | 21087-64-9 | 1.6E+03 | nc | 2.1E+04 | nc | 4.9E+02 | nc |
| Mirex | 2385-85-5 | 3.6E-02 3.4E-02 | c | 1.7E+00 | c | 8.8E-04 7.6E-04 | c |
| Molybdenum | 7439-98-7 | 3.9E+02 | nc | 5.8E+03 | nc | 1.0E+02 | nc |
| Monochloramine | 10599-90-3 | 7.8E+03 | nc | 1.2E+05 | nc | 2.0E+03 4.0E+03 | ncgws |
| Naled | 300-76-5 | 1.2E+02 | nc | 1.0E+03 1.1E+03 | nc | 1.1E+01 1.2E+01 | nc |
| Nickel and compounds | 7440-02-0 | 1.5E+03 1.4E+03 | nc | 2.2E+04 1.7E+04 | nc | 1.0E+02 | gws |
| Nitrate | 14797-55-8 | 1.3E+05 | nc | 1.0E+06 | max | 1.0E+04 | gws |
| Nitrite | 14797-65-0 | 7.8E+03 | nc | 1.2E+05 | nc | 1.0E+03 | gws |
| 2-Nitroaniline | 88-74-4 | 6.3E+02 | nc | 8.0E+03 | nc | 1.9E+02 | nc |
| Nitrobenzene | 98-95-3 | 5.5E+00 4.8E+00 | c | 2.4E+02 | c | 1.4E-01 1.2E-01 | c |
| Nitrofurantoin | 67-20-9 | 4.4E+03 | nc | 5.7E+04 | nc | 1.4E+03 | nc |
| Nitrofurazone | 59-87-0 | 4.2E-01 4.0E-01 | c | 1.8E+01 | c | 6.0E-02 5.3E-02 | c |
| Nitroglycerin | 55-63-0 | 6.3E+00 | nc | 8.2E+01 | nc | 2.0E+00 | nc |
| 2-Nitropropane | 79-46-9 | 1.5E-02 5.9E-02 | c | 6.4E-01 3.0E+00 | c | 2.1E-03 8.4E-03 | c |
| N-Nitrosodi-n-butylamine | 924-16-3 | 1.0E-01 9.4E-02 | c | 4.7E+00 | c | 2.7E-03 2.4E-03 | c |
| N-Nitrosodiethanolamine | 1116-54-7 | 1.9E-01 1.8E-01 | c | 8.2E+00 | c | 2.8E-02 2.5E-02 | c |
| N-Nitrosodiethylamine | 55-18-5 | 8.1E-04 8.0E-04 | c | 1.5E-01 | c | 1.7E-04 1.6E-04 | c |
| N-Nitrosodimethylamine | 62-75-9 | 2.0E-03 | c | 3.5E-01 | c | 1.1E-04 | c |
| N-Nitrosodiphenylamine | 86-30-6 | 1.1E+02 | c | 4.7E+03 | c | 1.2E+01 1.1E+01 | c |
| N-Nitroso di-n-propylamine | 621-64-7 | 7.8E-02 7.4E-02 | c | 3.3E+00 | c | 1.1E-02 9.5E-03 | c |
| N-Nitroso-N-methylethylamine | 10595-95-6 | 2.0E-02 1.9E-02 | c | 9.4E-01 | c | 7.1E-04 6.2E-04 | c |
| N-Nitrosopyrrolidine | 930-55-2 | 2.6E-01 2.5E-01 | c | 1.1E+01 | c | 3.7E-02 3.3E-02 | c |
| m-Nitrotoluene | 99-08-1 | 6.3E+00 | nc | 8.2E+01 | nc | 1.7E+00 | nc |
| o-Nitrotoluene | 88-72-2 | 3.2E+00 3.0E+00 | c | 1.5E+02 | c | 3.1E-01 2.8E-01 | c |
| p-Nitrotoluene | 99-99-0 | 3.4E+01 3.2E+01 | c | 1.4E+03 | c | 4.3E+00 3.8E+00 | c |
| NuStar | 85509-19-9 | 1.3E+02 | nc | 1.6E+03 | nc | 3.1E+01 | nc |
| Oryzalin | 19044-88-3 | 7.0E+01 6.6E+01 | c | 2.9E+03 | c | 7.9E+00 7.0E+00 | c |
| Oxadiazon | 19666-30-9 | 3.2E+02 7.3E+00 | ncc | 4.1E+03 3.2E+02 | ncc | 4.7E+01 4.4E-01 | ncc |
| Oxamyl | 23135-22-0 | 1.6E+03 | nc | 2.1E+04 | nc | 2.0E+02 | gws |
| Oxyfluorfen | 42874-03-3 | 7.4E+00 7.1E+00 | c | 3.1E+02 | c | 5.4E-01 4.8E-01 | c |
| Paraquat dichloride | 1910-42-5 | 2.8E+02 | nc | 3.7E+03 | nc | 9.0E+01 | nc |
| Parathion | 56-38-2 | 3.8E+02 | nc | 4.9E+03 | nc | 8.6E+01 | nc |
| Pentachlorobenzene | 608-93-5 | 5.0E+01 | nc | 5.0E+02 | nc | 2.0E+00 | nc |
| Pentachloronitrobenzene | 82-68-8 | 2.7E+00 2.6E+00 | c | 1.3E+02 | c | 1.2E-01 1.1E-01 | c |
| Pentachlorophenol | 87-86-5 | 1.0E+00 9.6E-01 | c | 4.0E+01 | c | 1.0E+00 | gws |
| Per- and Polyfluoroalkyl Substances (PFAS) |  |  |  |  |  |  |  |
| Perfluorooctanesulfonic acid (PFOS) | 1763-23-1 | 6.3E-03 | nc | 8.2E-02 | nc | 4.0E-03 | gws |
| Perfluorooctanoic acid (PFOA) | 335-67-1 | 1.8E-05 | c | 7.8E-04 | c | 4.0E-03 | gws |
| Perchlorate and perchlorate salts | Various | 5.5E+01 | nc | 8.2E+02 | nc | 1.4E+01 | nc |
| Permethrin | 52645-53-1 | 3.2E+03 | nc | 4.1E+04 | nc | 1.0E+03 | nc |
| Phenol | 108-95-2 | 1.9E+04 | nc | 2.5E+05 | nc | 5.8E+03 | nc |
| m-Phenylenediamine | 108-45-2 | 3.8E+02 | nc | 4.9E+03 | nc | 1.2E+02 | nc |
| p-Phenylenediamine | 106-50-3 | 6.3E+01 | nc | 8.2E+02 | nc | 2.0E+01 | nc |
| 2-Phenylphenol | 90-43-7 | 2.9E+02 2.7E+02 | c | 1.2E+04 | c | 3.1E+01 2.7E+01 | c |
| Phosphine | 7803-51-2 | 2.3E+01 | nc | 3.5E+02 | nc | 5.7E-01 | nc |
| Phosphorus (white) | 7723-14-0 | 1.6E+00 | nc | 2.3E+01 | nc | 4.0E-01 | nc |
| p-Phthalic acid | 100-21-0 | 3.2E+04 | nc | 4.1E+05 | nc | 9.4E+03 | nc |
| Phthalic anhydride | 85-44-9 | 1.3E+05 | nc | 1.0E+06 | max | 3.9E+04 | nc |
| Polybrominated biphenyls | 36355-01-8 | 1.8E-02 1.7E-02 | c | 7.7E-01 | c | 2.6E-03 2.3E-03 | c |
| Polychlorinated biphenyls (PCBs) | 1336-36-3 | 3.1E-01 2.9E-01 | c | 1.4E+01 | c | 5.0E-01 | gws |
| Aroclor 1016 | 12674-11-2 | 5.5E+00 | nc | 8.2E+01 | nc | 2.2E-01 2.0E-01 | c |
| Aroclor 1221 | 11104-28-2 | 2.6E-01 2.4E-01 | c | 1.2E+01 | c | 7.9E-03 6.8E-03 | c |
| Aroclor 1232 | 11141-16-5 | 2.2E-01 2.0E-01 | c | 1.0E+01 | c | 7.9E-03 6.8E-03 | c |
| Aroclor 1242 | 53469-21-9 | 3.1E-01 3.0E-01 | c | 1.5E+01 | c | 7.9E-03 6.8E-03 | c |
| Aroclor 1248 | 12672-29-6 | 3.1E-01 3.0E-01 | c | 1.5E+01 | c | 7.9E-03 6.8E-03 | c |
| Aroclor 1254 | 11097-69-1 | 3.2E-01 3.1E-01 | c | 1.5E+01 | c | 7.9E-03 6.8E-03 | c |
| Aroclor 1260 | 11096-82-5 | 3.3E-01 3.1E-01 | c | 1.6E+01 | c | 7.9E-03 6.8E-03 | c |
| Polycyclic Aromatic Hydrocarbons (PAHs) |  |  |  |  |  |  |  |
| Acenaphthene | 83-32-9 | 4.1E+03 4.2E+03 | nc | 4.7E+04 4.9E+04 | nc | 2.4E+02 2.6E+02 | nc |
| Acenaphthylene | 208-96-8 | 4.2E+03 4.3E+03 | nc | 5.1E+04 5.3E+04 | nc | 2.4E+02 2.6E+02 | nc |
| Anthracene | 120-12-7 | 2.3E+04 | nc | 3.5E+05 | nc | 1.8E+03 | nc |
| Benz[a]anthracene | 56-55-3 | 1.5E+00 | c | 3.2E+02 | c | 3.0E-02 2.8E-02 | c |
| Benzo[b]fluoranthene | 205-99-2 | 1.1E+00 | c | 2.1E+02 | c | 2.5E-01 2.4E-01 | c |
| Benzo[k]fluoranthene | 207-08-9 | 1.1E+01 | c | 2.1E+03 | c | 2.5E+00 2.4E+00 | c |
| Benzo[g,h,i]perylene | 191-24-2 | 1.8E+03 | nc | 2.3E+04 | nc | 6.0E+02 | nc |
| Benzo[a]pyrene | 50-32-8 | 1.1E-01 | c | 2.1E+01 | c | 2.0E-01 | gws |
| Chrysene | 218-01-9 | 1.1E+02 | c | 2.1E+04 | c | 2.5E+01 2.4E+01 | c |
| Dibenz[a,h]anthracene | 53-70-3 | 1.1E-01 | c | 2.1E+01 | c | 2.5E-02 2.4E-02 | c |
| Fluoranthene | 206-44-0 | 2.4E+03 | nc | 3.0E+04 | nc | 8.0E+02 | nc |
| Fluorene | 86-73-7 | 2.9E+03 | nc | 3.7E+04 3.8E+04 | nc | 1.5E+02 1.6E+02 | nc |
| Indeno[1,2,3-cd]pyrene | 193-39-5 | 1.1E+00 | c | 2.1E+02 | c | 2.5E-01 2.4E-01 | c |
| 1-Methylnaphthalene | 90-12-0 | 2.4E+01 1.3E+01 | c | 3.9E+02 | Csat | 1.1E+00 5.7E-01 | c |
| 2-Methylnaphthalene | 91-57-6 | 3.1E+02 | nc | 4.7E+03 | nc | 3.6E+01 | nc |
| Naphthalene | 91-20-3 | 2.4E+00 2.2E+00 | c | 1.1E+02 | c | 1.2E-01 1.0E-01 | c |
| Phenanthrene | 85-01-8 | 2.3E+04 | nc | 3.5E+05 | nc | 1.7E+03 | nc |
| Pyrene | 129-00-0 | 2.3E+03 | nc | 3.4E+04 | nc | 7.9E+01 8.1E+01 | nc |
| Prometon | 1610-18-0 | 9.5E+02 | nc | 1.2E+04 | nc | 2.5E+02 | nc |
| Prometryn | 7287-19-6 | 2.5E+03 | nc | 3.3E+04 | nc | 6.0E+02 | nc |
| Propachlor | 1918-16-7 | 8.2E+02 | nc | 1.1E+04 | nc | 2.5E+02 | nc |
| Propanil | 709-98-8 | 3.2E+02 | nc | 4.1E+03 | nc | 8.2E+01 | nc |
| Propargite | 2312-35-8 | 2.9E+00 2.7E+00 | c | 1.2E+02 | c | 1.6E-01 1.4E-01 | c |
| n-Propylbenzene | 103-65-1 | 2.6E+02 | Csat | 2.6E+02 | Csat | 6.6E+02 | nc |
| Propylene glycol | 57-55-6 | 1.0E+06 | max | 1.0E+06 | max | 4.0E+05 | nc |
| Propylene glycol, monoethyl ether | 1569-02-4 | 3.9E+04 | Csat | 3.9E+04 | Csat | 1.4E+04 | nc |
| Propylene glycol, monomethyl ether | 107-98-2 | 4.2E+04 | nc | 1.1E+05 | Csat | 3.2E+03 | nc |
| Pursuit | 81335-77-5 | 1.6E+05 | nc | 1.0E+06 | max | 4.7E+04 | nc |
| Pyridine | 110-86-1 | 5.8E+01 5.9E+01 | nc | 5.1E+02 5.5E+02 | nc | 5.3E+00 5.9E+00 | nc |
| Quinoline | 91-22-5 | 1.8E-01 1.7E-01 | c | 7.7E+00 | c | 2.4E-02 2.1E-02 | c |
| RDX (Cyclonite) | 121-82-4 | 6.1E+00 8.0E+00 | c | 2.8E+02 3.8E+02 | c | 7.0E-01 8.5E-01 | c |
| Resmethrin | 10453-86-8 | 1.9E+03 | nc | 2.5E+04 | nc | 6.7E+01 | nc |
| Ronnel | 299-84-3 | 3.8E+03 | nc | 5.1E+04 5.2E+04 | nc | 2.0E+02 2.1E+02 | nc |
| Rotenone | 83-79-4 | 2.5E+02 | nc | 3.3E+03 | nc | 6.1E+01 | nc |
| Selenious Acid | 7783-00-8 | 3.9E+02 | nc | 5.8E+03 | nc | 1.0E+02 | nc |
| Selenium | 7782-49-2 | 3.9E+02 | nc | 5.8E+03 | nc | 5.0E+01 | gws |
| Silver and compounds | 7440-22-4 | 3.9E+02 | nc | 5.8E+03 | nc | 9.4E+01 | nc |
| Simazine | 122-34-9 | 4.5E+00 4.3E+00 | c | 1.9E+02 | c | 4.0E+00 | gws |
| Sodium azide | 26628-22-8 | 3.1E+02 | nc | 4.7E+03 | nc | 8.0E+01 | nc |
| Sodium diethyldithiocarbamate | 148-18-5 | 2.0E+00 1.9E+00 | c | 8.5E+01 | c | 2.9E-01 2.5E-01 | c |
| Strontium, stable | 7440-24-6 | 4.7E+04 | nc | 7.0E+05 | nc | 1.2E+04 | nc |
| Strychnine | 57-24-9 | 1.9E+01 | nc | 2.5E+02 | nc | 5.9E+00 | nc |
| Styrene | 100-42-5 | 8.7E+02 | Csat | 8.7E+02 | Csat | 1.0E+02 | gws |
| tert-butanol | 75-65-0 | 1.4E+03 1.3E+03 | ncc | 2.1E+04 6.5E+04 | ncc | 3.6E+02 1.4E+02 | ncc |
| 2,3,7,8-Tetrachlorodibenzodioxin (TCDD/dioxin) | 1746-01-6 | 5.2E-06 4.9E-06 | c | 2.4E-04 | c | 3.0E-05 | gws |
| 1,2,4,5-Tetrachlorobenzene | 95-94-3 | 1.7E+01 2.3E+00 | nc | 1.5E+02 3.1E+01 | nc | 9.8E-01 1.6E-01 | nc |
| 1,1,1,2-Tetrachloroethane | 630-20-6 | 2.1E+00 1.9E+00 | c | 9.4E+01 | c | 5.7E-01 5.0E-01 | c |
| 1,1,2,2-Tetrachloroethane | 79-34-5 | 6.4E-01 5.7E-01 | c | 2.8E+01 | c | 7.6E-02 6.6E-02 | c |
| Tetrachloroethylene (PCE) | 127-18-4 | 2.5E+01 2.2E+01 | c | 1.7E+02 | Csat | 5.0E+00 | gws |
| 2,3,4,6-Tetrachlorophenol | 58-90-2 | 1.9E+03 | nc | 2.5E+04 | nc | 2.4E+02 | nc |
| p,a,a,a-Tetrachlorotoluene | 5216-25-1 | 2.1E-02 2.6E-02 | c | 9.8E-01 1.3E+00 | c | 5.5E-04 6.4E-04 | c |
| Tetrahydrofuran | 109-99-9 | 2.0E+04 | nc | 1.0E+05 | nc | 3.4E+03 | nc |
| Thallium and compounds | 7440-28-0 | 7.8E-01 | nc | 1.2E+01 | nc | 2.0E+00 | gws |
| Thiobencarb | 28249-77-6 | 6.3E+02 | nc | 8.2E+03 | nc | 1.6E+02 | nc |
| Thiocyanates | Various | 1.6E+01 | nc | 2.3E+02 | nc | 4.0E+00 | nc |
| Tin and compounds | 7440-31-5 | 4.7E+04 | nc | 7.0E+05 | nc | 1.2E+04 | nc |
| Toluene | 108-88-3 | 8.2E+02 | Csat | 8.2E+02 | Csat | 1.0E+03 | gws |
| Toluene-2,4-diamine | 95-80-7 | 1.4E-01 1.3E-01 | c | 6.0E+00 5.7E+00 | c | 2.0E-02 1.7E-02 | c |
| Toluene-2,5-diamine | 95-70-5 | 3.0E+00 2.9E+00 | c | 1.3E+02 | c | 4.3E-01 3.8E-01 | c |
| Toluene-2,6-diamine | 823-40-5 | 1.9E+03 | nc | 2.5E+04 | nc | 6.0E+02 | nc |
| p-Toluidine | 106-49-0 | 1.8E+01 1.7E+01 | c | 7.7E+02 | c | 2.5E+00 2.2E+00 | c |
| Toxaphene | 8001-35-2 | 4.9E-01 4.7E-01 | c | 2.1E+01 | c | 3.0E+00 | gws |
| 1,2,4-Tribromobenzene | 615-54-3 | 2.8E+02 2.9E+02 | nc | 2.4E+03 2.6E+03 | nc | 2.0E+01 2.2E+01 | nc |
| Tributyltin oxide (TBTO) | 56-35-9 | 1.9E+01 | nc | 2.5E+02 | nc | 5.7E+00 | nc |
| 2,4,6-Trichloroaniline | 634-93-5 | 1.9E+00 | nc | 2.5E+01 | nc | 4.0E-01 | nc |
| 1,2,4-Trichlorobenzene | 120-82-1 | 2.4E+01 2.3E+01 | c | 2.8E+02 | nc | 7.0E+01 | gws |
| 1,1,1-Trichloroethane | 71-55-6 | 6.4E+02 | Csat | 6.4E+02 | Csat | 2.0E+02 | gws |
| 1,1,2-Trichloroethane | 79-00-5 | 1.2E+00 1.1E+00 | c | 6.8E+00 | nc | 5.0E+00 | gws |
| Trichloroethylene (TCE) | 79-01-6 | 1.0E+00 9.2E-01 | c | 2.0E+01 | nc | 5.0E+00 | gws |
| Trichlorofluoromethane | 75-69-4 | 7.9E+02 | nc | 1.2E+03 | Csat | 1.1E+03 | nc |
| 2,4,5-Trichlorophenol | 95-95-4 | 6.3E+03 | nc | 8.2E+04 | nc | 1.2E+03 | nc |
| 2,4,6-Trichlorophenol | 88-06-2 | 4.9E+01 4.7E+01 | c | 8.2E+02 | nc | 4.1E+00 3.6E+00 | c |
| 2,4,5-Trichlorophenoxyacetic Acid | 93-76-5 | 6.3E+02 | nc | 8.2E+03 | nc | 1.6E+02 | nc |
| 2-(2,4,5-Trichlorophenoxy) propionic acid | 93-72-1 | 5.1E+02 | nc | 6.6E+03 | nc | 5.0E+01 | gws |
| 1,1,2-Trichloropropane | 598-77-6 | 1.7E+02 1.8E+02 | nc | 1.0E+03 1.1E+03 | nc | 2.6E+01 2.8E+01 | nc |
| 1,2,3-Trichloropropane | 96-18-4 | 5.1E-03 5.1E-03 | c | 1.1E+00 | c | 7.5E-04 7.2E-04 | c |
| 1,2,3-Trichloropropene | 96-19-5 | 7.8E-01 | nc | 3.3E+00 | nc | 6.2E-01 | nc |
| 1,1,2-Trichloro-1,2,2-trifluoroethane (Freon 113) | 76-13-1 | 9.1E+02 | Csat | 9.1E+02 | Csat | 1.0E+04 | nc |
| 1,2,4-Trimethylbenzene | 95-63-6 | 2.2E+02 | Csat | 2.2E+02 | Csat | 5.6E+01 | nc |
| 1,3,5-Trimethylbenzene | 108-67-8 | 1.8E+02 | Csat | 1.8E+02 | Csat | 6.0E+01 | nc |
| Trimethyl phosphate | 512-56-1 | 2.7E+01 2.6E+01 | c | 1.1E+03 | c | 3.9E+00 3.4E+00 | c |
| 1,3,5-Trinitrobenzene | 99-35-4 | 2.2E+03 | nc | 3.2E+04 | nc | 5.9E+02 | nc |
| Trinitrophenylmethylnitramine (Tetryl) | 479-45-8 | 1.6E+02 | nc | 2.3E+03 | nc | 3.9E+01 | nc |
| 2,4,6-Trinitrotoluene | 118-96-7 | 2.1E+01 2.0E+01 | c | 5.1E+02 | nc | 2.5E+00 2.2E+00 | c |
| Vanadium and compounds | 7440-62-2 | 4.6E+02 4.0E+02 | c | 8.4E+03 | nc | 1.5E+02 | nc |
| Vinclozolin | 50471-44-8 | 7.6E+01 | nc | 9.8E+02 | nc | 2.1E+01 | nc |
| Vinyl acetate | 108-05-4 | 9.7E+02 | nc | 2.7E+03 | Csat | 4.1E+02 | nc |
| Vinyl bromide | 593-60-2 | 2.8E-01 2.4E-01 | c | 1.2E+01 | c | 3.7E-01 3.2E-01 | c |
| Vinyl chloride | 75-01-4 | 6.1E-02 6.0E-02 | c | 1.8E+01 | c | 2.0E+00 | gws |
| Warfarin | 81-81-2 | 1.9E+01 | nc | 2.5E+02 | nc | 5.6E+00 | nc |
| Xylenes | 1330-20-7 | 2.6E+02 | Csat | 2.6E+02 | Csat | 1.0E+04 | gws |
| Zinc and Compounds | 7440-66-6 | 2.3E+04 | nc | 3.5E+05 | nc | 6.0E+03 | nc |
| Zinc phosphide | 1314-84-7 | 2.3E+01 | nc | 3.5E+02 | nc | 6.0E+00 | nc |
| Zineb | 12122-67-7 | 3.2E+03 | nc | 4.1E+04 | nc | 9.9E+02 | nc |
| Notes 1Where appropriate, the residential and industrial soil values consider ingestion and dermal exposure to soil and inhalation exposure to contaminants moving from soil to ambient air from volatilization or particulate emission. 2Groundwater standards promulgated under 47CSR12 are provided, where available. Standards that are unavailable under 47CSR12 are based on a risk-based methodology that considers ingestion, dermal, and inhalation exposure arising from the domestic use of groundwater. 3The concentrations in this table shall be applied where the exposure pathways described in footnotes 1 and 2 are the major contributors to risks identified in the site assessment. If other exposure pathways are identified, the acceptable concentrations shall be determined only in consultation with the Secretary, considering all exposure pathways, and all other requirements of the regulations. 4Basis of standard: c – cancer effect; nc – noncancer effect; max – calculated risk-based concentration exceeds maximum possible contaminant level of 1x106 mg/kg; Csat – calculated risk-based concentration exceeds residual saturation level; gws – West Virginia Groundwater Quality Standards from 47CSR12. *Lead – Residential soil based on Revised Interim Soil Lead Guidance for CERCLA Sites and RCRA Corrective Action Facilities (July 1994), U.S. EPA OSWER Directive 9355.4-12. Industrial soil based on the U.S. EPA’s Blood Lead Concentrations of U.S. Adult Females: Summary Statistics from Phases 1 and 2 of the National Health and Nutrition Evaluation Survey (NHANES III) (2002) and Lead at Superfund Sites: Frequent Questions from Risk Assessors on the Adult Lead Methodology (2018). |  |  |  |  |  |  |  |